# 04 – How Computers Talk: TCP/IP Model vs OSI Model

Professional Internship Learning Material

Both models describe how data travels from one computer to another, split into layers. Each layer does one job and talks to the layer above and below it.

## OSI Model (7 Layers)

| # | Layer | Job | Examples / Protocols | Data Unit |
| --- | --- | --- | --- | --- |
| 7 | Application | Interface for apps/users | HTTP, DNS, SMTP, FTP | Data |
| 6 | Presentation | Format, encrypt, compress | TLS/SSL, JPEG | Data |
| 5 | Session | Open, manage, close sessions | NetBIOS, RPC | Data |
| 4 | Transport | End-to-end delivery, ports | TCP, UDP | Segment / Datagram |
| 3 | Network | Addressing and routing | IP, ICMP, routers | Packet |
| 2 | Data Link | Local delivery using MAC addresses | Ethernet, Wi-Fi, switches | Frame |
| 1 | Physical | Actual bits over cable/radio | Cables, hubs, signals | Bits |

Memory trick (top to bottom): All People Seem To Need Data Processing.

## TCP/IP Model (4 Layers)

| Layer | Job | Protocols |
| --- | --- | --- |
| Application | User-facing services | HTTP/S, DNS, SMTP, FTP, SSH |
| Transport | Reliable or fast delivery | TCP, UDP |
| Internet | Addressing and routing | IP, ICMP, ARP |
| Network Access (Link) | Physical + local delivery | Ethernet, Wi-Fi |

## Mapping Between Them

| OSI | TCP/IP |
| --- | --- |
| Application, Presentation, Session | Application |
| Transport | Transport |
| Network | Internet |
| Data Link, Physical | Network Access |

## Comparison

| Point | OSI | TCP/IP |
| --- | --- | --- |
| Layers | 7 | 4 |
| Nature | Theoretical reference model | Practical model the internet actually uses |
| Developed by | ISO | US DoD / ARPANET |
| Approach | Protocol-independent | Built around specific protocols |
| Used for | Teaching, troubleshooting | Real communication |

## TCP vs UDP (Transport Layer)

|  | TCP | UDP |
| --- | --- | --- |
| Connection | Connection-oriented (3-way handshake: SYN, SYN-ACK, ACK) | Connectionless |
| Reliability | Reliable, ordered | Best effort |
| Speed | Slower | Faster |
| Used for | Web, email, file transfer | Streaming, DNS, gaming |

## Encapsulation

Data moves down the layers on the sender side, with each layer adding a header, and up on the receiver side, with each layer removing it.

## Why It Matters for a SOC

Analysts read logs and packets from different layers: firewall logs (L3/L4), web logs (L7), switch logs (L2). Knowing the layer tells you what kind of attack or fault you are looking at.

Document 04 • Prepared for Internship Submission